SECTION 14202

ELEVATORS AND LIFTS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2008 - 2017 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Savaria ; residential and commercial elevators and lifts.
 .
 This section is based on the products of Savaria, which is located at:
2 Walker Drive  
Brampton, ON, Canada L6T 5E1
Toll Free Tel: 800-661-5112
Tel: 905-791-5555
Fax: 905-791-2222
Email: request info
Web: www.savaria.com

As a leading designer and manufacturer of residential elevators, luxury home elevators, commercial elevators and lifts for over 30 years, Savaria Lifts has thousands of satisfied customers to thank for their business.
 Our global operation continuously seeks to advance manufacturing processes in our own modern facilities, allowing us to offer exceptional value and shorter lead times, while never compromising product quality, safety or reliability.
 As an active member of many trade associations, we are committed to the ongoing support of its customers and dealers alike. Authorized dealers are carefully selected for their knowledge and experience, kept up-to-date with the latest technical and training sessions, and supported with our expert customer service.

- GENERAL

  - SECTION INCLUDES

    - Residential elevators.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant for elevator products; add others as required.

    - Division 16 Sections for electrical service for elevators to and including disconnect and fused switches at machine room.

    - Division 16 Sections for standby power source, transfer switch, and connection from auxiliary contacts in transfer switch to controller.

    - Division 16 Section "Voice and Data Communication Cabling" for telephone service to elevators.

    - Section 03300 - Cast-in-Place Concrete: Concrete for elevator machine foundation, and pit.

    - Section 06100 - Rough Carpentry: Hoistway framing, building-in hoistway door frames and overhead hoist beams.

    - Section 08210 - Wood Doors: Hoistway doors.

    - Section 08710 - Door Hardware.

    - Section 09260 - Gypsum Board Assemblies: Gypsum shaft walls.

    - Section 09650 - Resilient Flooring: Floor finish in cab.

    - Section 09686 - Carpet: Floor finish in cab.

    - Section 09900 - Paints and Coatings: Interior transparent wood finish in cab.

    - Section 13850 - Detection and Alarm: Fire and smoke detectors and interconnecting devices.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not required.

    - American National Standards Institute (ANSI) B-29.2 - Chain Standards for Inverted Tooth (Silent) Chains and Sprockets.

    - American Society of Mechanical Engineers (ASME) A17.1 - Safety Code for Elevators and Escalators.

    - American Society of Mechanical Engineers (ASME) A18.1 - Safety Standard for Platform and Stairway Chair Lifts.

    - CSA B44.1 - Elevator and Escalator Electrical Equipment.

    - CSA B355 - Lifts for Persons with Physical Disabilities.

    - CSA B613 - Private Residence Lifts for Persons with Physical Disabilities.

    - U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)".

    - ICC/ANSI A117.1 - Accessible and Usable Buildings and Facilities.

    - NFPA 70 - National Electric Code.

    - CSA - National Electric Code.

  - REQUIREMENTS OF REGULATORY AGENCIES:

    - Fabricate and install work in compliance with applicable jurisdictional authorities.

    - File shop drawings and submissions with local authorities as the information is made available. Company pre-inspection and jurisdictional authority inspections and permits are to be made on timely basis as required.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Provide a complete layout of lift equipment detailing dimensions and clearances as required.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

    - Selection Samples: For each finish product specified requiring selection of color or finish, two complete sets of color charts representing manufacturer's full range of available colors and patterns.

  - QUALITY ASSURANCE

    - Installer Qualifications:

      - Skilled tradesmen shall be employees of the installing contractor approved by the manufacturer, with demonstrated ability to perform the work on a timely basis.

      - Execute work of this section only by a company that has adequate product liability insurance.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install systems under environmental conditions outside manufacturer's absolute limits.

  - WARRANTY

    - Coverage - this warranty applies to the repair or replacement, at Manufacturer's option, of parts that fail due to defective material or workmanship. Manufacturer may, at its option, provide factory reconditioned parts. This warranty is provided to the Authorized Dealer on behalf of the final purchaser of the product and is not transferable. The Manufacturer's warranty does not cover labor charges for the removal, repair or replacement of warranty parts but such costs may be covered for a period of time by Authorized Dealer's warranty, which is provided to purchaser separately.

      - The manufacturer shall offer a 36-month limited warranty on parts from date of shipment.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:  Savaria, which is located at: 2 Walker Drive, Brampton, ON, Canada, L6T 5E1; Toll Free Tel: 800-661-5112; Tel: 905-791-5555; Fax:  905-791-2222; Email: request info; Web: www.savaria.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

  - RESIDENTIAL ELEVATOR

    - Residential Elevator: Savaria Telecab , Telecab17

      - Model:

** NOTE TO SPECIFIER ** Please select model type; Please note; Telecab has 500 lb (227 kg) weight capacity and Telecab17 has 845 lb (383 kg) weight capacity

        - Telecab

        - Telecab17

    - Equipment: Provide equipment, incidental material and labor required for complete, operable hydraulic elevator installation. The elevators shall be erected, installed, adjusted, tested and placed in operation by the elevator system manufacturer or manufacturer's authorized installer.

    - Performance: The elevator shall be designed and tested in accordance with ASME A17.1 part V. The testing shall consist of loading the platform to rated capacity for several cycles to insure proper operation. Mechanical failures and defects shall be corrected.

    - The following preparatory work to receive the elevator specified shall be the work provided by others:

      - Dedicated 110 VAC 20 amp single phase power to operate lift to be provided.

      - No pit shall be provided.

      - Provide a load-bearing wall to support a load of:

** NOTE TO SPECIFIER ** Please Select load bearing wall weight and delete option not required. For Telecab17 option ‘b’ must be selected

        - 700 lbs at any point.

        - 950 lbs at any point.

      - Provide a floor cut out. Refer to shop drawings.

      - The floor needs to support 3200 lb (1452 kg).

      - A phone line shall be provided.

    - Characteristics:

      - Rated load:

** NOTE TO SPECIFIER ** Please Select rated load and delete option not required. For Telecab model option ‘a’ must be selected. For Telecab17 model, option ‘b’ must be selected

        - 500 lb (227 kg).

        - 845 lb (383 kg)

      - Rated Speed: 20 fpm. (0.10 m/s).

      - Car Dimensions:

** NOTE TO SPECIFIER ** Delete platform size not required. For Telecab model, option ‘a’ must be selected. For Telecab17 model, select option’b’ or ‘c’

        - 30 inches W by 46 inches L by 78 inches H (762 mm by 1168 mm by 1981 mm).

        - 30 inches W by 46 inches L by 80 inches H (762 mm by 1168 mm by 2032 mm).

        - 36 inches W by 54 inches L by 80 inches H (914 mm by 1372 mm by 2032 mm).

      - Levels Serviced: 2.

      - Door Operation

** NOTE TO SPECIFIER ** Select type of door operation and delete option not required. For Telecab17 model, option ‘b’ must be selected

- Manual door.

- Hydraulic door closer.

- Automatic door operator.

      - Cab Configuration: Enter/exit same side.

** NOTE TO SPECIFIER ** Please enter travel height

      - Travel: ______ feet. Maximum of 23 feet (276 inches).

      - Operations: Constant pressure.

      - Power Supply: 110 VAC, 20 amps, 1 Phase.

      - Drive System: 2 to 1 roller chain hydraulic.

      - Paint: Powder coat finish.

      - Color: White.

      - Emergency Power:

** NOTE TO SPECIFIER ** Select type of emergency operation below and delete option not required.

- Battery operation in down direction (standard).

- 24 VDC battery back up in both up and down direction (optional)

      - Controller: Relay logic based controller.

      - Motor/Pump: 3 HP/geared type (standard)

      - Car Operating Panel: Constant pressure buttons, emergency stop/alarm button, on/off key switch, emergency light, and an alarm button mounted on a removable stainless steel panel.

      - Hall Call Stations: Provide flush or surface mounted landing call/send stations.

      - Emergency Operation: The car shall be equipped with battery-operated emergency lighting. The battery shall be the rechargeable type with an automatic recharging system. A manual lowering device shall be located in a lockable box positioned in the machine room.

    - Car Enclosure:

      - Walls: Metal wall panels.

      - Ceilings: Metal with two recessed pot lights.

      - Floor: Metal with non-skid surface.

      - Handrail location: Handrail located on the door side.

      - Clear plexiglass windows will be provided on cab walls as required.

      - Car door dimensions and type

** NOTE TO SPECIFIER ** Select car door dimensions and delete option not required. For Telecab model, option ‘a’ must be selected. For Telecab17 model, option ‘b’ or ‘c’ must be selected

        - 78 inches H by 30 inches W (1981 mm by 762 mm) and shall be made of steel with a plexiglass insert.

        - 80 inches H by 30 inches W (2032 mm by 762 mm) and shall be made of aluminum tubing with a plexiglass insert.

        - 80 inches H by 36 inches W (2032 mm by 914 mm) and shall be made of aluminum tubing with a plexiglass insert.

    - Systems and Components:

      - The pumping unit and control shall be enclosed in the tower. The controller and pump unit shall be pre-wired and tested prior to shipment. Pump unit shall incorporate the following features:

        - Adjustable pressure relief valve.

        - Gate valve to isolate cylinder from pump unit.

        - Electrical solenoid for down direction control.

      - The cylinder shall be constructed of steel pipe of sufficient thickness and suitable safety margin. The top of the cylinder shall be equipped with a cylinder head with an internal guide ring and self-adjusting packing.

      - The plunger shall be constructed of a steel shaft of proper diameter machined true and smooth. The plunger shall be provided with a stop electrically welded to the bottom to prevent the plunger from leaving the cylinder.

      - Roller Chains: Two No.50 roller chains with 5/8 inch (16 mm) pitch. Minimum breaking strength 6100 lb (2772 kg) each.

      - Leveling Device: The lift shall be provided with an anti-creep device which will maintain the carriage level within 1/2 inch (13 mm) of the top landing.

        - All limit switches and leveling device switches shall be located in a position to be inaccessible to unauthorized persons. The switches shall be located behind the mast wall and be accessible through removable panels.

      - Guide Yoke: The 2:1 guide yoke/idler guides arrangements shall be supplied with two idler guides, roller guide shoes, bearings and guards.

      - Terminal Stopping Device: Normal terminal stopping devices shall be provided at top and bottom of runway to stop the car positively and automatically. Limit switches shall be used.

      - Guide Rail and Brackets: Steel 'C'' guide rails and brackets shall be used to guide the platform and sling. Guide rails shall form part of the structural integrity of the unit and be integral to the mast enclosure, ensuring stability and minimum platform deflection when loaded.

      - Wiring and electrical connections shall comply with applicable codes. Insulated wiring shall have flame-retardant and moisture-proof outer covering and shall be run in conduit or electrical wire way if located outside the unit enclosure. Quick disconnect harnesses shall be used when possible.

      - The door lock shall be electrical mechanical weather resistant WR-500 CSA approved elevator lock.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until hoistway and machine room has been properly prepared.

    - Site dimensions shall be taken to verify that tolerances and clearances have been maintained and meet local regulations.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

** NOTE TO SPECIFIER ** Delete if not required.

  - ELEVATOR INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install the components of the elevator system that are required and that are required by jurisdictional authorities to license the elevator.

    - Trained employees of the elevator contractor shall perform installation work.

    - Adjust elevator for proper operation and clean unit thoroughly.

    - Instruct users in operating procedures and owner's maintenance person in trouble-shooting and maintenance procedures.

** NOTE TO SPECIFIER ** Delete if not required.

  - LIFT INSTALLATION

    - Install all the components of the lift system that are specified in this section to be provided, and that are required by jurisdictional authorities to license the lift.

    - Trained employees of the lift contractor shall perform all installation work of this section.

    - Adjust lift for proper operation and clean unit thoroughly.

    - Instruct users in operation procedures and Owner's maintenance person in trouble-shooting and maintenance procedures.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION