SECTION 14202

ELEVATORS AND LIFTS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2008 - 2008 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Savaria ; residential and commercial elevators and lifts.
 .
 This section is based on the products of Savaria, which is located at:
2 Walker Drive  
Brampton, ON, Canada L6T 5E1
Toll Free Tel: 800-661-5112
Tel: 905-791-5555
Fax: 905-791-2222
Email: request info
Web: www.savaria.com  
 
As a leading designer and manufacturer of residential elevators, luxury home elevators, commercial elevators and lifts for over 30 years, Savaria Lifts has thousands of satisfied customers to thank for their business.
 Our global operation continuously seeks to advance manufacturing processes in our own modern facilities, allowing us to offer exceptional value and shorter lead times, while never compromising product quality, safety or reliability.
 As an active member of many trade associations, we are committed to the ongoing support of its customers and dealers alike. Authorized dealers are carefully selected for their knowledge and experience, kept up-to-date with the latest technical and training sessions, and supported with our expert customer service.
Commercial LU/LA Elevator: 
Savaria provides user-friendly, good value solutions for both new construction, and retrofitting into existing buildings. 
Savaria Orion LU/LA: A fully enclosed commercial elevator that makes code compliance fast and easy. Completely redesigned, it meets all current American and Canadian Code requirements for limited-use/ limited application (LU/LA) elevators. It is also suitable for residential use.

- GENERAL

  - SECTION INCLUDES

    - Commercial  LU/LA elevator.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant for elevator products; add others as required.

    - Division 16 Sections for electrical service for elevators to and including disconnect and fused switches at machine room.

    - Division 16 Sections for standby power source, transfer switch, and connection from auxiliary contacts in transfer switch to controller.

    - Division 16 Section "Voice and Data Communication Cabling" for telephone service to elevators.

    - Section 03300 - Cast-in-Place Concrete: Concrete for elevator machine foundation, and pit.

    - Section 06100 - Rough Carpentry: Hoistway framing, building-in hoistway door frames and overhead hoist beams.

    - Section 08210 - Wood Doors: Hoistway doors.

    - Section 08710 - Door Hardware.

    - Section 09260 - Gypsum Board Assemblies: Gypsum shaft walls.

    - Section 09650 - Resilient Flooring: Floor finish in cab.

    - Section 09686 - Carpet: Floor finish in cab.

    - Section 09900 - Paints and Coatings: Interior transparent wood finish in cab.

    - Section 13850 - Detection and Alarm: Fire and smoke detectors and interconnecting devices.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not required.

    - American National Standards Institute (ANSI) B-29.2 - Chain Standards for Inverted Tooth (Silent) Chains and Sprockets.

    - American Society of Mechanical Engineers (ASME) A17.1 - Safety Code for Elevators and Escalators.

    - American Society of Mechanical Engineers (ASME) A18.1 - Safety Standard for Platform and Stairway Chair Lifts.

    - CSA B44.1 - Elevator and Escalator Electrical Equipment.

    - CSA B355 - Lifts for Persons with Physical Disabilities.

    - CSA B613 - Private Residence Lifts for Persons with Physical Disabilities.

    - U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)".

    - ICC/ANSI A117.1 - Accessible and Usable Buildings and Facilities.

    - NFPA 70 - National Electric Code.

    - CSA - National Electric Code.

  - REQUIREMENTS OF REGULATORY AGENCIES:

    - Fabricate and install work in compliance with applicable jurisdictional authorities.

    - File shop drawings and submissions with local authorities as the information is made available. Company pre-inspection and jurisdictional authority inspections and permits are to be made on timely basis as required.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Provide a complete layout of lift equipment detailing dimensions and clearances as required.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

    - Selection Samples: For each finish product specified requiring selection of color or finish, two complete sets of color charts representing manufacturer's full range of available colors and patterns.

  - QUALITY ASSURANCE

    - Installer Qualifications:

      - Skilled tradesmen shall be employees of the installing contractor approved by the manufacturer, with demonstrated ability to perform the work on a timely basis.

      - Execute work of this section only by a company that has adequate product liability insurance.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install systems under environmental conditions outside manufacturer's absolute limits.

  - WARRANTY

    - Coverage - this warranty applies to the repair or replacement, at Manufacturer's option, of parts that fail due to defective material or workmanship. Manufacturer may, at its option, provide factory reconditioned parts. This warranty is provided to the Authorized Dealer on behalf of the final purchaser of the product and is not transferable. The Manufacturer's warranty does not cover labor charges for the removal, repair or replacement of warranty parts but such costs may be covered for a period of time by Authorized Dealer's warranty, which is provided to purchaser separately.

      - The manufacturer shall offer a 36-month limited warranty on parts from date of shipment.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:  Savaria, which is located at: 2 Walker Drive, Brampton, ON, Canada, L6T 5E1; Toll Free Tel: 800-661-5112; Tel: 905-791-5555; Fax:  905-791-2222; Email: request info; Web: www.savaria.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

  - COMMERCIAL PASSENGER ELEVATOR

** NOTE TO SPECIFIER ** Delete model not required.

    - Limited Use Limited Application elevator model:

- Savaria Orion - Hydraulic

- 2:1 roped hydraulic drive.

- Duty cycle: normal 30 trips per day, heavy 75 trips per day with a maximum number of start per hour on a standard installation of 15x

- Savaria Orion – MRL

- 1:1 counterweight traction with direct drive gearbox and safety gear. Using a 6 pole, 3 phase motor to reduce noise level. Direct acting 1:1 ratio on the sheave to eliminates rope run noise. The gearbox shall be mounted on nitrile anti-vibration mounts for further noise reduction.  Roller guide shall be use on the cab sling and guide shoes on the counterweight to further reduce noise.

- Duty cycle : 200 trips per day 45 max starts per hour

    - Work described in this section includes providing equipment, incidental material and labor required for complete, operable roped hydraulic passenger elevator installation. Elevator shall be erected, installed, adjusted, tested and placed in operation by system manufacturer, or manufacturer's authorized installer.

      - Elevators shall be in accordance with the ASME A17.1-B44 and ADA compliant including local codes and regulations except where specified otherwise.

    - The following preparatory work to receive the lifts specified in this section is part of the work of other sections:

      - Permanent 240 VAC, 40 amp single phase or 208 VAC, 30 amp three phase power to operate lift to be provided from a lockable fused/cartridge type disconnect switch with auxiliary contacts for battery operation. 110 VAC, 15 amp single phase power to operate the lighting circuit. Refer to drawings for permanent power specifications and location of disconnects.

      - Provide a plumb and square hoistway with smooth interior surfaces, including fascias or furring of the hoistway interior.

      - Provide rough openings per lift contractor's shop drawings.

      - Provide substantial, level pit floor slab as indicated on the lift contractor's shop drawings.

    - Limited Use Limited Application elevator: The elevator described here, manufactured by Savaria Lifts Inc., is a LULA Elevator consisting of:

- Rated Load: 1400 lb (635 kg)

- Rated Speed: 30 f.p.m. (nominal) (0.15 m/s)

** NOTE TO SPECIFIER ** Please enter travel height

- Travel: ______ feet. Maximum of 25 feet (300 inches).

- Cab Configuration:

** NOTE TO SPECIFIER ** Delete cab configurations not required. Option ‘a’ or ‘b’ must be chosen for Orion17 model. Option ‘c’ must be chosen for Orion model.

- Enter/exit same side.

- Enter/exit front/rear.

- 90 degree exit.

- Car Platform Size:

** NOTE TO SPECIFIER ** Select platform size required and delete other options. The 51” x 51” platform size must and can only be used on a 90 degree platform configuration. The 42” x 54” cab is only available with the Orion MRL model.

- 42” W by 54” D (1067 mm by 1371 mm)

- 48” W by 54” D (1219 mm by 1371 mm)

- 42" W by 60” D (1067mm by 1524 mm)

- 51” W by 51”D (1295 mm by 1295 mm)

- Levels Serviced:

** NOTE TO SPECIFIER ** Delete levels not required.

- 2

- 3

- 4.

- Car Operation: Automatic.

- Power Supply:

** NOTE TO SPECIFIER ** Delete option for power supply not required.

- 208 Volt, 3 Phase, 30 Amps + 110 Volt, 15 amp, 1 Phase 60 Hz

- 240 Volt, 1 Phase, 40 Amp + 110 Volt, 15 amp, 1 Phase 60 Hz

- Emergency Power: Battery operation in down direction

- Controller: PLC

- Manual Lowering: Outside the hoist way in machine room or via access hatch for MRL.

    - Elevator Cab Design:

** NOTE TO SPECIFIER ** Select car enclosure design required. Either the Orion 17 car or Orion STD car.

- Orion 17 Car Enclosure: Steel cab with standard PLAM overlays & stainless steel dropped ceiling and trims.

- Cab Walls: Laminate Overlay with Stainless Steel Trim

- Plastic Laminate Overlay Finish:

** NOTE TO SPECIFIER ** Delete plastic laminate overlay options not required

- Reclaimed Wood

- Gray

- Walnut

- Ceiling Finish: Stainless Steel Drop Down Ceiling with a combination of hidden panel lighting and LED pot lights

- Car doors and frames shall be 1 1/2 hour ULC Fire rated and 2 speed horizontally sliding. Car door finish shall be Stainless Steel brushed #4

- Handrail: A stainless steel single handrail, with 1-1/2 inch (38 mm) diameter rail shall be located on the control wall of the cab.

** NOTE TO SPECIFIER ** Delete Orion car enclosure section if not required.

- Orion STD Car Enclosure: Steel or s/s wall construction with optional raised PLAM panels.

- Cab Walls:

** NOTE TO SPECIFIER ** Delete walls options not required.

- Steel - Architectural White (standard)

- Steel - Black (Standard)

- Stainless Steel Brushed #4 (optional)

- Optional Raised Plastic Laminate or Raised Melamine Panels:

** NOTE TO SPECIFIER ** Delete plastic laminate and melamine options not required. Please note that the steel cab option can only be used with an architectural white painted cab.

- No Rasied Panels - Steel Cab Only (standard)

- Stone Grafix (Plastic Laminate)

- Fog (Plastic Laminate)

- Mahogany (Plastic Laminate)

- Natural Oak (Plastic Laminate)

- Cherry (Melamine)

- Candlelight (Melamine)

- Silken Maple (Melamine)

- Antique White (Melamine)

- Ceiling Finish:

** NOTE TO SPECIFIER ** Delete ceiling option not required.

- Steel - Architectural White with four recessed incandescent down lights.

- Stainless Steel brushed #4 (optional)

- Car doors and frames shall be 1 1/2 hour ULC Fire rated and 2 speed horizontally sliding.

Door finish shall be:

** NOTE TO SPECIFIER ** Delete car door finish not required.

- Powder coated Architectural white or black to match cab finish

- Stainless Steel brushed #4 (optional)

Handrail: A stainless steel single handrail, with 1-1/2 inch (38 mm) diameter rail shall be located on the control wall of the cab.

    - Appendix E package including: directional arrow and voice annunciator.

** NOTE TO SPECIFIER ** Select whether appendix E package is required.

- Yes

- No

    - Automatic Landing Doors

- Landing doors and frames shall be 1 1/2 hour ULC Fire rated, 2 speed horizontally sliding with concealed mechanical interlock.

Door finish shall be:

** NOTE TO SPECIFIER ** Delete landing door finish not required.

- Primed powder coated grey (Standard)

- Powder coated Architectural white or black to match cab finish    (optional)

- Stainless Steel brushed #4 (optional)

    - Car Operation:

      - Car Operating Panel shall consist of metal push bottoms with illuminated haloes, tactile identifications, emergency stop/alarm button, on/off key switch and emergency light mounted on a removable stainless steel panel (Type 304 #4 Stainless Steel Finish).

      - Digital floor indicator and directional indicator in cab and at each landing.

      - An ADA hands free phone will be supplied within car operating panel.

      - Emergency Operation - The car shall be equipped with a battery operated light fixture, emergency battery lowering device and alarm in case of normal building supply failure. The battery shall be the rechargeable type with an automatic recharging system. A manual lowering device shall be located outside the hoistway in the machine room.

      - Fire Service:

** NOTE TO SPECIFIER ** Delete fire-service options not required.

- No fire-service required (standard)

- Phase 1 fire recall service only (optional) - mandatory for 2010 A17.1 code.

- Phase 1 and Phase 2 fire recall service (optional).

    - Drive System:

** NOTE TO SPECIFIER ** Delete drive system not applicable. Either select hydraulic 2:1 roped drive or 1:1 counterweight traction with direct drive.

- Hydraulic 2:1 roped drive

- Pumping Unit and Controller: The pumping unit and controller shall be in a separate machine room. The controller and pump unit shall be pre-wired and tested prior to shipment. Pump unit shall incorporate the following features:

- Smooth stops at each landing.

- Submersible pump and motor.

- Adjustable pressure relief valve.

- Manually operable down valve to lower lift in the event of an emergency. This valve shall be activated from the machine room.

- Gate valve to isolate cylinder from pump unit.

- Emergency lowering by battery power from the car control

- Cylinder And Plunger:

- The cylinder shall be constructed of steel pipe of sufficient thickness and suitable safety margin. The top of the cylinder shall be equipped with a cylinder head with an internal guide ring and self-adjusting packing.

- The plunger shall be constructed of a solid steel shaft of proper diameter machined true and smooth. The plunger shall be provided with a stop electrically welded to the bottom to prevent the plunger from leaving the cylinder

- Cable: Aircraft Cable 2 X 3/8'' (10 mm) DIA. Minimum breaking strength of 12,000 lb (5455 kg) each.

- Guide Yoke: The 2:1 guide yoke assembly shall be supplied with one (1) sheave, guide shoes, bearings and guards.

- Guide Rails and Brackets: Steel 8 lb per ft guide rails and adjustable brackets shall be used to guide the platform and sling.

- Motor/Pump: 240 1 Phase or 208 3 Phase,/ 5HP

- 1:1 counterweight traction with direct drive gearbox and safety gear.

- Controller location:

** NOTE TO SPECIFIER ** Select controller location. Either controller will be remote in a machine room space or will be provided in a cabinet that is accessed via the landing and mounted next to one of the door entrances.

- TRUE MRL with controller in door buck

    - at landing level:____________________

- In remote machine room

- Drive system and controller:

- Motor: 6 pole 3 phase motor coupled to a 13:1 product specific gearbox to reduce noise level.

- The drive unit shall incorporate the following features:

- Smooth stops at each landing.

- Emergency lowering by battery power

- Guide rail system

- Steel 8lb per ft guide rails shall be used for guide rails and counterweight rails. Roller guide shall be used on the cab sling and guide shoes on the counterweight to further reduce noise

- Cable:  Elevator traction cable 3 x 3/8” (10mm) DIA 8 x19 sealed with natural fiber core, regular lay. Minimum breaking strength 8,200 lb (3,727 kg) each.

    - Leveling Device:

      - The lift shall be provided with an anti-creep device which will maintain the carriage level within 1/2 inch (12 mm) of each landing.

      - All limit switch and leveling device switches shall be located in a position to be inaccessible to unauthorized persons.

    - Terminal Stopping Devices: Normal terminal stopping devices shall be provided at top and bottom of runway to stop the car positively and automatically.

    - Wiring: All wiring and electrical connections shall comply with applicable codes. Insulated wiring shall have flame-retardant and moisture-proof outer covering and shall be run in conduit or electrical wire ways if located outside the unit enclosure. Quick disconnect harnesses shall be used when possible.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until hoistway and machine room has been properly prepared.

    - Site dimensions shall be taken to verify that tolerances and clearances have been maintained and meet local regulations.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

** NOTE TO SPECIFIER ** Delete if not required.

  - ELEVATOR INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install the components of the elevator system that are required and that are required by jurisdictional authorities to license the elevator.

    - Trained employees of the elevator contractor shall perform installation work.

    - Adjust elevator for proper operation and clean unit thoroughly.

    - Instruct users in operating procedures and owner's maintenance person in trouble-shooting and maintenance procedures.

** NOTE TO SPECIFIER ** Delete if not required.

  - LIFT INSTALLATION

    - Install all the components of the lift system that are specified in this section to be provided, and that are required by jurisdictional authorities to license the lift.

    - Trained employees of the lift contractor shall perform all installation work of this section.

    - Adjust lift for proper operation and clean unit thoroughly.

    - Instruct users in operation procedures and Owner's maintenance person in trouble-shooting and maintenance procedures.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION